Advance Computer Network Assignment 4

Submitted by: Aarav Kaim

Router Configuration 
After creating the topology, we’ll click router for configuration.

Router  CLI:

Router 1 >

en

Router #

config t

Router (config) #

interface Fa0/0

ip address 192.168.10.1 255.255.255.0

no shut

ip dhcp pool xyz

Router (dhcp-config) 3#

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 1.2.3.4

lease 0 3

exit

Client Configuration

Click each PC:

Desktop  IP Configuration  click:

DHCP

It should auto-receive:

- IP (like 192.168.10.x)

- Mask

- Gateway

- DNS