Advance Computer Network Assignment 7

Submitted by: Aarav Kaim

Router 5 Configuration 
After creating the topology, we’ll click router for configuration.

Router 5  CLI:

en

config t

interface Se2/0

ip address 100.1.1.1 255.255.255.0

no shut

interface Se3/0

ip address 20.1.1.1 255.255.255.0

no shut

router ospf 1

router-id 1.1.1.1

network 100.1.1.0 0.0.0.255 area 0

network 20.1.1.0 0.0.0.255 area 0

end

Router 6 Configuration

Router 6  CLI:

en

config t

interface Se2/0

ip address 100.1.1.2 255.255.255.0

no shut

interface Fa0/0

ip address 192.16.2.1 255.255.0.0

no shut

router ospf 1

router-id 2.2.2.2

network 100.1.1.0 0.0.0.255 area 0

network 192.16.2.0 0.0.255.255 area 0

end

Router 7 Configuration

Router 7  CLI:

en

config t

interface Se3/0

ip address 20.1.1.2 255.255.255.0

no shut

interface Fa0/0

ip address 10.2.2.1 255.0.0.0

no shut

router ospf 1

router-id 3.3.3.3

network 20.1.1.0 0.0.0.255 area 0

network 10.2.2.0 0.255.255.255 area 0end

Clients Configuration 
PC  Desktop  ip configuration

PC4:

IP  192.16.2.1

Mask  255.255.0.0

PC5:

IP  10.2.2.20

Mask  255.0.0.0