Advance Computer Network Assignment 8

Submitted by: Aarav Kaim

Router 0 Configuration 
After creating the topology, we’ll click router for configuration.

Router 0  CLI:

en

config t

interface Se2/0

ip address 10.0.0.1 255.255.255.252

no shut

interface Fa0/0

ip address 192.168.1.1 255.255.255.0

no shut

router ospf 1

router bgp 100

neighbor 10.0.0.2 remote-as 200

network 192.168.1.0 mask 255.255.255.0

end

Router 1 Configuration

Router 1  CLI:

en

config t

interface Se2/0

ip address 10.0.0.2 255.255.255.252

no shut

interface Se3/0

ip address 10.0.0.5 255.255.255.252

no shut

router bgp 200

neighbor 10.0.0.1 remote-as 100

neighbor 10.0.0.6 remote-as 300

network 10.0.0.0 mask 255.255.255.252

end

Router 2 Configuration

Router 2  CLI:

en

config t

interface Se3/0

ip address 10.0.0.6 255.255.255.252

no shut

interface Fa0/0

ip address 192.168.3.1 255.255.255.0

no shut

router bgp 300

neighbor 10.0.0.5 remote-as 200

network 192.168.3.0 mask 255.255.255.0

end

Clients Configuration 
PC  Desktop  ip configuration

PC0:

IP  192.168.1.2

Mask  255.255.255.0

PC1:

IP  192.168.3.2

Mask  255.255.255.0